From Giuseppe (and Walter)

DEM quality check

The ENO-DEM90 present several negative values ranging from -600 to 0. There are 723 tiles with negative values, distributed all over the world. Some of them are “true” negative values representing a real situation of the earth surface under the see level (e.g. Dead Sea, Death Valley). Others can be considered dem voids.

Figure 1: Geographical location of tiles having negative values.

Figure 2: Scatterplot between the minimum values at tile level of ENO-DEM90 and the 1km GTOPO30.  All the tiles that are not close to the 45 degree relationship can be considered having artefacts situation.

Figure 3:  Scatterplot between the maximum values at tile level of ENO-DEM90 and the 1km GTOPO30. All the tiles that are not close to the 45 degree relationship can be considered having artefacts situation. In blue and in green tiles having a difference in the maximum value larger than 1000 meters.

Figure 4: Geographical location of tiles having a difference in the maximum value larger than 1000 meters (blue and green cross in the Figure 3).

Correction of negative values

I implement a correction of the negative values in order to reduce the number of classes (every 100 meters) in all the tiles, with a final aim of creating a 100 threshold probability maps. Indeed, the presence of negative values creates classified tiles also in please where I was sure that there were not negative values.

I come up with a solution that fit my objectives and was quite easily to implement in one week. The correction has been performed only on those tiles having negative numbers. Nonetheless, voids having values over zero were not considered effect so much the final results, and the chain processing speed.  All the pixels of the ENO-DEM90 tiles having negative values have been replaced with the values of 1kmGTOPO30. In this way I was sure that all the negative values were replaced with values close to the average in 1km grid resolution.

In Figure 2 and 4 I was plotting only minimum and maximum value at tile level, nonetheless similar errors are also presented in the range between minimum and maximum. A scatterplot of all the pixel in one single tile can highlight voids and spikes.

An additional flag that i would consider in the distribution of the ENO-DEM90 is to label the sea area as no_data value [e.g. -9999], in order to easy mask out. If the sea is label as zero you need un ancillary layer to mask out the sea. Such a label is also present in the GTOPO30.